REQUIRED LABORATORY HAZARD ASSESSMENT SURVEY

OSHA Standards 29 CFR 1910. 132-138

Personal Protective Equipment Requirements*

Department:

Lab rooms:

P.I.:

Person completing survey (P.I./Lab Manager only):			  Signature:

Date:

Please complete Assessment as soon as possible, file the original, & post a copy in the lab.

| Check If Applicable | Hazards in the Workplace | PPE Required |
| --- | --- | --- |
|  | HAZARDOUS LIQUID CHEMICALS |  |
| _ _ | Corrosive Liquids (Acids/Caustics): Small containers < 1 liter Large containers > 1 liter | Safety goggles preferred, safety glasses accepted, gloves, long sleeved lab coat, closed toe shoes. If work is done on an open bench and not in a chemical fume hood, it is advisable to use goggles instead of glasses. Safety goggles, gloves, long sleeved lab coat, rubber apron, closed toe shoes. If potential for a splash is high, it is advisable to use both face shield and goggles. Also if work is done on an open bench and not in a chemical fume hood, it is recommended that a face shield be used in addition to goggles. |
| _ _ | Flammable Liquids Dispensing from 5 gal drums | Safety goggles preferred, safety glasses accepted, gloves, long sleeved lab coat, closed toe shoes. Safety goggles, gloves, long sleeved lab coat, closed toe shoes. If potential for a splash is high it is advisable to use a face shield in addition to goggles. |
| _ | Highly reactive liquid chemicals and high energy oxidizers | Safety goggles preferred, safety glasses accepted, long sleeved lab coat, gloves, closed toe shoes. Face shield or body shield must be used in addition to protective eyewear during the reaction based on the scale of the reaction. |
| _ | Liquids with high acute toxicity (poisons) | Safety goggles, gloves, long sleeved lab coat, impermeable apron, closed toe shoes. If potential for a splash high use impermeable coveralls and a face shield in addition to goggles. |
| _ | Liquids with high chronic toxicity (known and suspected carcinogens, reproductive toxins) | Safety goggles preferred, safety glasses accepted, gloves, long sleeved lab coat, closed toe shoes. |
| _ | Other hazardous liquid chemicals (not included in the above categories) | Safety goggles/glasses based on a splash potential, gloves, long sleeved lab coat, closed toe shoes. |
|  | SOLID CHEMICALS |  |
| _ | Caustic solids (lime, etc) | Safety glasses, gloves, long sleeved lab coat, closed toe shoes. If potential for getting a chemical into the face is high use face shield in addition to glasses. |
| _ | Flammable solids (alkali metals, red phosphorous, etc) | Safety glasses, gloves, long sleeved lab coat, closed toe shoes. |
| _ | Highly reactive solids & high energy oxidizers | Safety glasses, gloves, long sleeved lab coat, closed toe shoes. Face shield or body shield during reaction based on the scale of the reaction in addition to protective eyewear. |
| _ | Solids of high acute toxicity (poisons) | Safety glasses, gloves, long sleeved lab coat, closed toe shoes. If must weigh, use particulate mask. If potential for disseminating powder high, use protective coveralls and a face shield in addition to glasses and mask. |
| _ | Solids of high chronic toxicity (carcinogens and reproductive toxins) | Safety glasses, gloves, long sleeved lab coat, closed toe shoes. If must weigh, use particulate mask.or vented balance enclosure. |
| _ | Other hazardous solid chemicals (not included above) | Safety glasses, long sleeved lab coat, gloves, closed toe shoes. |
| _ | Compressed toxic gases | Safety goggles, gloves, long sleeved lab coats, closed toe shoes. |
| _ | Work with pressurized glass/plastic vessels (potential for creating flying fragments) | Face shield, safety goggles/glasses based on substances in the vessel, long sleeved lab coat, gloves, closed toe shoes. |
|  | LIGHT RADIATION |  |
| _ _ _ _ _ | Welding: Electric arc Welding: Gas Cutting, torch brazing, torch soldering Lasers Class IIIb or IV Sources of UV light (hand hold irradiators, UV screens, germicidal lamps) | Welding helmet or shield, gloves, long sleeved lab coat, closed toe shoes. Welding goggles or face shield, gloves, long sleeved lab coat, closed toe shoes. Spectacles or welding face shield, gloves, long sleeved lab coat, closed toe shoes. Protective eyewear of the appropriate optical density Safety glasses, long sleeved lab coat, gloves. Based on the duration of exposure and the source it may be advisable to use a face shield instead of glasses. |
|  | TEMPERATURE EXTREMES: |  |
| _ _ _ | Heat (hot surfaces, hot solutions, etc) Cold Low temperature freezers Cryogenic gases (N2, He2, etc) | Face shield, thermal gloves, long sleeved lab coat, closed toe shoes. Thermal gloves, long sleeved lab coats, closed toe shoes. Face shield, thermal gloves, long sleeved lab coat, closed toe shoes. |
| _ | Autoclave operation | Face shield, rubber apron, thermal gloves, lab coat, closed toe shoes. |
| _ | Other hazards not listed above |  |

*	The personal protective equipment that appears on the survey has been assigned to provide protection for eyes, face, feet, and hands.  For additional information on PPE consult Page 10D "PersonalProtective Equipment" of the University Chemical Safety and Disposal Guide.  Use "Chemical Resistance Chart" on page12D for proper glove selection.  Contact Section of Radiation Safety at 2628769 when selecting laser protective eyewear.

**	Radioactive and biological materials are excluded from this lab hazard assessment survey because these issues are evaluated through special Radiation and Biological Safety Programs.  Radiological and biological hazards are not covered by the above listed OSHA standards and are subject to other Federal Regulations.

***	Respiratory protection is not included.  Respiratory protection is administered through the University Respiratory Protection Program by the staff of the Safety Department.