User’s Manual

Annual Phosphorus Loss Estimator

Version 2026

3/1/2026

Peter A. Vadas (developer of APLE)

peter.vadas@usda.gov

USDA-ARS

Office of National Programs

Beltsville, MD, USA

Carl H. Bolster (developer of APLE2026)

carl.bolster@usda.gov

USDA-ARS

Food Animal Environmental Systems Research Unit

Bowling Green, KY, USA

Contents

Model Description	3

Download and Install Program	4

Running APLE2026	4

1.	Populate Input Data File Worksheets	5

fieldData	5

rainData	7

fertilizerData	7

liquidManureData	8

solidManureData	10

grazingManureData	11

observedData	12

2.	Select Runoff Method and Soil Layers	12

3.	Load Data	12

4.	Input Mehlich-3 and Clay data	12

5.	Run APLE	12

Viewing Results	13

Saving Results	14

Clearing Results	14

# Model Description

The Annual P Loss Estimator (APLE) model is an annual time-step model for predicting annual field-scale surface runoff and erosion losses of dissolved and particulate P as well as annual changes in total and Mehlich-3 extractable soil P. APLE was originally developed by Vadas et al. (2009) and later updated by Bolster and Vadas (Bolster & Vadas, 2022). APLE is intended to be user-friendly and does not require extensive input data to operate. It is intended to estimate edge-of-field P loss for uniform fields of several hectares in size, or smaller. APLE does not simulate P loss through grassed waterways or buffers that may occur beyond the field edge. The model includes routines to account for P applications in the form of applied animal manure; highly soluble commercial fertilizers such as superphosphate, triple superphosphate, or mono- and di-ammonium phosphate; and dung deposited by grazing cattle. APLE has been shown to produce reasonably good predictions of both annual field-scale P loss and changes in soil test P at a variety of sites (Bolster et al., 2017; Bolster et al., 2022; Vadas et al., 2012). The model has been used to evaluate the effect of different management practices on P loss and soil P levels (Mott et al., 2025; Withers et al., 2019) and to evaluate and improve P site indices – a simple tool used by many states to evaluate risks of P loss and to guide in P management strategies to protect water quality from P runoff from agricultural fields (Bolster, 2011; Bolster et al., 2014; Fiorellino et al., 2017).

APLE was originally developed and coded as an Excel workbook to provide a modeling option for users with limited modeling experience and lack of access to expensive software packages. The advantage of using Excel is that most users have both access and familiarity with Excel. However, coding within Excel is very difficult to follow as there are numerous calls to multiple cells making it challenging to modify and update. Moreover, the graphics are limited, and the current version does not have the ability to compare model predictions with observations. This limits the model’s use and functionality. To address this, we have developed a Graphical User Interface (GUI) application of APLE (APLE2026) to provide a cleaner and more intuitive user interface, improved graphics, and greater output capabilities.

# Download and Install Program

APLE2026 was coded and compiled in MATLAB 2025b as a stand-alone application that does not require a MATLAB software license. The executable file, template input file (APLE_Input_Data.xlsx), and a modified version of MATLAB Runtime which allows users without MATLAB to run the app are available in a single .zip file (APLE2026_Install.zip ) for download from the APLE Home Page (www.ars.usda.gov/APLE).

After downloading APLE2026_Install.zip, extract the files to destination folder. Once extracted, run CustomRuntimeInstaller.exe and follow the prompts. This installs MATLAB Runtime giving the user the ability to run APLE2026 without a MATLAB license. Once completed, run MyAppInstaller.exe and follow the prompts. The final step is to move the input files into the APLE directory.

# Running APLE2026

## Populate Input Data File Worksheets

Most data needed to run the model are read from an Excel workbook. It is recommended to use the accompanying Excel template input file (APLE_Input_Data.xlsx). Users can modify the file name, but sheet names and column names must remain the same. Included in the download is a template input file (APLE_Input_Data.xlsx). The spreadsheet includes the following worksheets for users to supply data to run the model: fieldData, rainData, fertilizerData, solidManureData, liquidManureData, grazingManureData, and observedData. Details of each worksheet are described below. The Years columns in each sheet can either be populated with consecutive years (i.e. starting at 1 and incrementing by 1 for each year of the simulation) or calendar years but must be consistent across data sheets. Units must also be the same as those listed in input file.

### fieldData

Variable Name		Definition

Depth of Soil layers (in)	The model is intended to simulate processes in only the topsoil, and can simulate two layers in the topsoil. This is intended to estimate P stratification (i.e., significantly different P concentrations in different soil layers) in soils with no or limited tillage. This can be important for pastures or no-till soils where P might accumulate in the top 1 inch of soil. The depth in inches to the bottom of each layer should be entered. APLE will return error messages if the top layer is less than 1 inch or the bottom layer is not deeper than the top layer.

If only one layer is to be simulated, columns related to the 2nd layer can be left blank.

Soil Bulk Density (g/cm3)	Soil bulk density is used to calculate total P in each layer and thus the P concentrations. Including soil bulk density as a user-defined input variable is in response to findings by Bolster et al. (2022) that predictions of Mehlich-3 soil P were relatively sensitive to bulk density values, particularly for stratified soils.

If only one layer is to be simulated, columns related to the 2nd layer can be left blank.

Soil OM (%)	This is the organic matter content in % of the soil layer(s). These data are used to estimate the PSP value that controls soil P transformations. It is therefore important to have accurate measurements of soil organic matter.

If only one layer is to be simulated, columns related to the 2nd layer can be left blank.

Crop P Export (lb/ac)	Enter the total annual crop P export in lb/acre. This is P that is actually removed from the field in harvested products. For typical row crops with high yields, this value may be 25 lb/acre. For double crops, the value may be greater, while for poorly yielding crops or pastures, the value may be only 5-10 lb/acre.

Sediment Loss (tons/ac)	Enter the total annual erosion rate in ton/acre. The user has to estimate this value. Well established pastures may lose only 0.1 ton/acre of soil, while highly erosive soils may lose 7-8 tons/acre of soil per year.

Curve Number	Two options exist for calculating annual runoff. One option is to estimate runoff from recorded daily or event precipitation data using the SCS Curve Number (CN). If using the CN method, a curve number must be entered for each year. The CN is assumed to be a function of soil group, land cover, and antecedent moisture condition. Published tables of CNs are available for a wide range of soil types and land uses/covers (neh630ch9cov (usda.gov)). If using the curve number method for calculating annual runoff, the precipData sheet must include daily rainfall amounts for each year of the model simulations. Columns for annual rain and annual runoff can be left blank if using the CN method.

Annual Rain (in)	If annual runoff is entered manually, input annual precipitation (rain, snow, and irrigation) in inches.

Annual Runoff (in)	Enter annual runoff in inches.

Degree of Soil Mixing (%)	This variable determines how much the two soil layers are mixed together each year, either by tillage or by natural processes, and also how much P added in fertilizer or manure gets mixed into the second soil layer. For pastures or no-till soils, there is still soil mixing even though no tillage occurs. A value such as 10-15% should be entered for such soils. For tilled soils, Table 1 lists values for common tillage equipment. If multiple tillage implements are used in a year, a value should be entered for the tillage operation that causes the most mixing. If the deepest depth of tillage is less than the depth of the first soil layer, then a value that is appropriate for no-till soils should be entered. Practically, this value should never be 0. If only simulating 1 soil layer this column can be left blank.

### rainData

Daily rainfall amounts for each year are added to the corresponding column. These data are only required when using the CN method for estimating annual runoff. Make sure that the years listed in first row match the years in the fidelData worksheet

### fertilizerData

Variable Name		Definition

Year	Year of fertilizer application. If there is more than one application per year, place each application on separate rows using same year. Only include years with a fertilizer application and do not skip rows. For instance, in the example below there were two separate fertilizer applications in 2012, none in 2013, and one application in 2014-2018.

Fertilizer P2O5 Applied	Enter the total amount of commercial fertilizer P2O5 applied in lb/acre. Be careful to enter the amount of P2O5 applied and not the amount of P applied (P is 44% of P2O5) or the amount of fertilizer mass applied.

Fertilizer Incorporated (%)	Enter the % of fertilizer incorporated into the soil either by injection or by tillage. Table 1 at the end of the document lists values for common tillage equipment.

Depth of Incorporation (in)	Enter the depth in inches to which fertilizer is directly injected or tilled into the soil.

### liquidManureData

Variable Name		Definition

Year	Year of liquid manure application. Liquid manure is defined as manures having solids content of < 15%. If more than one application per year, place each application in a separate row using same year. Only include years with a liquid manure application and do not skip rows. See example in fertilizerData worksheet description above.

Manure Applied (gal/ac)	Enter the application rate in gallons/acre for liquid manures. For liquid manures, the rate of application will determine how much of the manure P infiltrates into soil at application. This will reduce the amount of manure P that will be lost in runoff, so it is important to enter an accurate value.

Manure P2O5 (lbs/1000 gal)	Enter the total P2O5 content of the manure applied in lbs/1000 gallons. Be sure to enter P2O5 content and not P content. Total P is typically measured by digestion of a manure sample, such as Kjeldahl digestion. If measured data are not available, typical manure total P contents can be taken from literature sources such as Kleinman et al. (2005).

Manure WEP/TP (%)	This is the % of manure total P that is in a water-extractable form (WEP). For this model, manure WEP can be measured by shaking fresh manure with de-ionized water at a dry weight equivalent, water to manure solids ratio of 250:1 for 1 h, filtering extracts through 0.45-um filters, and measuring P in filtrates colorimetrically. Manure water extraction data are becoming more common (Bundy et al., 2004; Kleinman et al., 2005; Vadas and Kleinman, 2006), but most extractions will not use the procedure required for our model, particularly the 250:1 extraction ratio. However, data from any extraction ratio can be extrapolated to estimate manure P at a 250:1 ratio using regression figures relating extraction ratio to extractable P in Vadas et al. (2005). Default values can be used, including 50% for dairy and beef manure, 20% for poultry manure, 35% for swine manure, and 10% for manures amended to reduce soluble P (such as poultry litter amended with alum).

Manure Injection (yes/no)	Select from dropdown menu either yes or no to indicate if liquid manure is injected without tillage. APLE determines how much of the manure P actually enters the soil based on application rate, with the idea that liquid manures cannot be effectively injected at high rates (e.g., 25000 gallons per acre) and the manure P essentially remains on the soil surface.

Manure Incorporated (%)	Enter the % of total manure applied that is incorporated into the soil by tillage. Table 1 at the end of the document lists values for common tillage equipment.

Depth of Incorporation (in)	Enter the depth in inches to which manure is directly injected or tilled into the soil. Enter the deeper depth if injection and tillage both occur, although this is unlikely.

Season of Application	Select season that manure was applied (spring, summer, fall, winter) from dropdown menu. The rate of organic P mineralization calculated by APLE varies depending on season. Also, applications in the fall affect P loss estimates in the following year.

### solidManureData

Variable Name		Definition

Year	Year of solid manure application. Solid manure is defined as manures having solids content of > 15%. If there is more than one application per year, place each application in separate row using same year. Only include years with a solid manure application and do not skip rows. See example in fertilizerData worksheet description above.

Manure Applied (ton/ac)	Enter the solid manure application rate in wet ton/acre.

Manure P2O5 	Enter the total P2O5 content of the manure applied in lbs/wet ton. Be sure to enter P2O5 content and not P content. Total P is typically measured by digestion of a manure sample, such as Kjeldahl digestion. If measured data are not available, typical manure total P contents can be taken from literature sources such as Kleinman et al. (2005).

Manure WEP/TP (%)	This is the % of manure total P that is in a water-extractable form (WEP). For this model, manure WEP can be measured by shaking fresh manure with de-ionized water at a dry weight equivalent, water to manure solids ratio of 250:1 for 1 h, filtering extracts through 0.45-um filters, and measuring P in filtrates colorimetrically. Manure water extraction data are becoming more common (Bundy et al., 2004; (Kleinman et al., 2005; Vadas and Kleinman, 2006), but most extractions will not use the procedure required for our model, particularly the 250:1 extraction ratio. However, data from any extraction ratio can be extrapolated to estimate manure P at a 250:1 ratio using regression figures relating extraction ratio to extractable P in Vadas et al. (2005). Default values can be used, including 50% for dairy and beef manure, 20% for poultry manure, 35% for swine manure, and 10% for manures amended to reduce soluble P (such as poultry litter amended with alum).

Manure Incorporated (%)	Enter the % of total manure applied that is incorporated into the soil by tillage. Table 1 at the end of the document lists values for common tillage equipment.

Depth of Incorporation (in)	Enter the depth in inches to which manure is directly injected or tilled into the soil. Enter the deeper depth if injection and tillage both occur, although this is unlikely.

Season of Application	Select season that manure was applied (spring, summer, fall, winter) from dropdown menu. The rate of organic P mineralization calculated by APLE varies depending on season. Also, applications in the fall affect P loss estimates in the following year.

### grazingManureData

Variable Name		Definition

Year	Year of cattle grazing. Only include years with grazing cattle and do not skip rows.

Field Area (ac)	This is the area in acres of the field being simulated. Field area is only used when calculating manure application from grazing animals. If no grazing animals are present, the value entered is not critical. However, the cell should not be left blank or set to 0 because that will cause errors in calculations.

Grazing Animals	Enter the total number of grazing animal days for each type of cow. For example, if 50 lactating cows grazed for 20 days, the value entered should be 1000 (50x20) in Milk Cows column.

### observedData

Variable Name		Definition

Year	Year that data were measured. Only include years with measured data and do not skip rows.

## Select Runoff Method and Soil Layers

After opening APLE2026, users must select runoff method and number of soil layers to be modeled prior to loading data. If 2 layers are selected, ensure that data for the second soil layer is populated in the input file.

## Load Data

Select Load from the Files dropdown menu in the upper left corner of the app. Only .xlsx and .xls files are compatible with the app.

## Input Mehlich-3 and Clay data

Enter initial values of Mehlich-3 equivalent soil test P and clay content for soil layer(s) in the user interface.

Enter the soil test P content in ppm for each soil layer. The model is currently designed based on Mehlich-3 soil extraction data. Other soil test data should be converted to a Mehlich-3 equivalent. For these modeling purposes, it can be assumed that Mehlich-3 P is equal to Bray-1 P, and twice as much as Mehlich-1 P, Olsen P, Fe-oxide strip extractable P, and anion exchange resin extractable P.

Enter the clay content in % for each soil layer. Clay content is used to estimate the PSP value which controls how soil P is partitioned between labile, active, and stable pools. It is therefore important to have accurate measurements of soil clay. However, testing of the model has shown some inaccurate soil P predictions for low clay soils (<10%) that still have significant P sorption capacity, such as sandy soils with significant amounts of iron or aluminum. In these cases, soil clay should be set at about 10% to make sure the model still accounts for adequate P sorption.

## Run APLE

Press Run APLE2026 button in middle left of app.

## Viewing Results

In Results tab, the first tab (P Loss Plots) shows model predictions of annual dissolved, particulate, and total P loss. The second tab (Soil Mehlich-3 P Plots) shows model predictions of Mehlich-3 equivalent soil P concentrations for each soil layer as well as the average concentration for the two layers (if selected) combined. The third tab (Total Soil P Plots) shows model predictions of total soil P concentrations for each soil layer as well as the average concentration for the two layers (if selected) combined. To add measured data to any of these figures, select the Plot Obs Data button. To remove data from these figures, select Plot Obs Data button again.

In the fourth tab (P Loss Sources), a bar chart shows the sources of P loss (sediment, soil, fertilizer, and manure) that the model predicts. This figure is intended to help guide management decisions based on the model’s predictions of the major sources and pathways of P loss.

In the fifth tab (P drainage), the annual P loss from each soil layer is presented in bar charts. These calculations should only be used for informational purposes as these routines have yet to be evaluated against observed data.

To save any of these plots, select the Popout Plot button and select save from drop down menu.

In the sixth tab (Data Analysis), the user can compare model predictions with observations where available. Only tabs that are enabled can be selected. Once selected, a plot of observed vs predicted appears along with the following goodness-of-fit statistics: Root mean square error (RMSE), mean absolute error (MAE), Nash-Sutcliffe Efficiency (NSE), and bias. With 4 or more observations, a linear regression is fit to the data with the slope and intercept, along with their standard errors, are provided on the figure. By pushing the save plot button, the user can save a snapshot of the figure as a .jpg or .png file.

Model results are also provided in tabular form in the Predicted Data Table tab. In the Warning ID column in the Predicted Data Table, the following represent the following warnings:

1 – User-defined plant P removal exceeds model simulated plant available P in the top soil layer.

2 – User-defined plant P removal exceeds model simulated plant available P in the bottom soil layer.

3 – User-defined plant P removal exceeds model simulated plant available P in both soil layers.

## Saving Results

To save data in the Results Table, select Save from the Files dropdown menu.

Enter the sheet name for results (default is APLE_results) and select OK.

Select File or enter name of new file. If existing file is selected it will add existing results to file with selected sheet name and keep other sheets that are named differently (select Yes when an error message states that existing file will be replaced).

A message will alert user if data were successfully saved.

## Clearing Results

Results from a simulation can be cleared by pressing the Clear Results button.